ELECTRIC VEHICLE

CHARGE CONTROLLER FIRMWARE

PROGRAMMING GUIDE

EV 13.172

HUGHES PROPRIETARY

Copyright © Hughes Aircraft Company (1994-1996)

- unpublished work - All Rights Reserved

HUGHES POWER CONTROL SYSTEMS

PO BOX 2923

TORRANCE, CA 90509-2923

12 April 96

1. Introduction

1.1 Software Installation

2. Glossary

2.1 Customer.h Data File

2.2 Sensed Inputs

2.3 Outputs

2.4 Notes

3. Modes of Operation

3.1 Mode 0 (Power Up Delay)

3.2 Mode 2 (Constant Voltage #1)

3.3 Mode 2 (Constant Voltage #2)

3.4 Mode 3 (Traction Pack Leveling - Constant Voltage)

3.5 Mode 4 (Auxiliary Battery Charging - Constant Voltage)

3.6 Mode 254 (Completion Mode)

3.7 Mode 255 (Fault Mode - Safety - Applicable During All Modes)

4. Filters

5. Stability Considerations

6. Temperature Compensation

7. State of Charge

8. Brown-Out Conditions

9. Monitoring (Via RS232 Port)

10. Auxiliary Battery Protection

11. Dolphin Protection

12. Customer.h Equates File

12.1 Calibration Settings

12.2 Battery Information

12.2.1 Type of Battery

12.2.2 Battery (BATTERIES)

12.2.3 Strings (STRINGS)

12.2.4 Cells (CELLS)

12.2.5 Ampere Hours (AMP_HOURS)

12.2.6 Battery Maximum Voltage (VMAX)

12.2.7 Battery Minimum Voltage (VMIN)

12.3 Power Up Delay (PUD).

12.4 Time Maximum (TMAX)

12.5 Mode Parameter Settings

12.5.1 Mode 1 - Constant Voltage #1

12.5.1.1 Mode 1 Voltage Set Point (V1)

12.5.1.2 Mode 1 Current Trip Point (ITRIP1)

12.5.1.3 Mode 1 Maximum Current Allowed (IMAX_MODE1)

12.5.1.4 Bypass Mode 1 Trip Point (BYPASS_TRIPPT1)

12.5.1.5 Mode 1 Time in Hours and Minutes (T1_HOURS & T1_MIN)

12.5.2 Mode 2 - Constant Voltage #2

12.5.2.1 Mode 2 Voltage Set Point (V2)

12.5.2.2 Mode 2 Current Trip Point (ITRIP2)

12.5.2.3 Mode 2 Maximum Current Allowed (IMAX_MODE2)

12.5.2.4 Bypass Mode 2 Trip Point (BYPASS_TRIPPT2)

12.5.2.5 Mode 2 Time in Hours and Minutes (T2_HOURS & T2_MIN)

12.5.3 Mode 3 - Traction Pack Leveling - Constant Voltage

12.5.3.1 Mode 3 Voltage Set Point (V3)

12.5.3.2 Mode 3 Current Trip Point (ITRIP3)

12.5.3.3 Mode 3 Maximum Current Allowed (IMAX_MODE3)

12.5.3.4 Bypass Mode 3 Trip Point (BYPASS_TRIPPT3)

12.5.3.5 Mode 3 Time in Hours and Minutes (T3_HOURS & T3_MIN)

12.5.4 Mode 4 - Auxiliary Battery Charging - Constant Voltage

12.5.4.1 Mode 4 Voltage Set Point (V4)

12.5.4.2 Mode 4 Current Trip Point (ITRIP4)

12.5.4.3 Mode 4 Maximum Current Allowed (IMAX_MODE4)

12.5.4.4 Bypass Mode 4 Trip Point (BYPASS_TRIPPT4)

12.5.4.5 Mode 4 Time in Hours and Minutes (T4_HOURS & T4_MIN)

12.6 Temperature

12.6.1 Temperature Sensor Monitoring (MON_TEMP_SENSE)

12.6.2 Temperature Maximum (TEMPMAX).

12.6.3 Temperature Minimum (TEMPMIN).

12.6.4 Temperature Compensation Flag (TEMP_COMP_FLAG)

12.6.5 Temperature Compensation Maximum Voltage (TEMP_MAX_VOLTS)

12.6.6 Temperature Compensation Minimum Voltage (TEMP_MIN_VOLTS)

12.6.7 Temperature Compensation Millivolts (TEMP_COMP_MV)

12.6.8 Temperature Nominal Degrees Celsius (TEMP_NOMINAL_C)

12.7 Error Gain Factors

12.7.1 Current Error Gain Factor (I_COR)

12.7.2 Voltage Error Gain Factor (V_COR)

12.8 Special Installation Values and Flags

12.8.1 Amp Hour Maximum Limit (AMPHR_MAXLIMIT)

12.8.2 Dolphin Monitoring (MON_DOLPHIN)

12.8.3 Auxiliary Battery Monitoring (MON_AUX_BATT)

12.8.4 Perform Battery Leveling (DO_TOP_OFFS)

12.8.5 Level Traction Pack After n Cycles (LEVEL_AFTER)

13. Making Firmware Changes

13.1 Hardware Requirements

13.2 Software Requirements

13.3 Procedure

14. Programming the Micro-controller

14.1 Hardware Requirements

14.2 Software Requirements

14.3 Procedure

15. Charge Control Algorithm Worksheet

16. Inventory listing of the Installation Distribution Media

17. Point of Contact

# Introduction

EV13 is written as a generic charging control program.  It has two constant voltage operating modes that can be customized to meet each customer's needs. This can be accomplished by setting constants and variables found within the Customer.h file.  If a particular mode is not desired, it can be nulled out by setting the mode time parameters, usually to zero.  This should become clear in the following operational description of this program.

This charging control algorithm is basically described in various modes.  Mode transition is sequential except for Fault Mode (255) which can be entered directly from any mode.  Completion Mode may be exited when traction pack leveling and/or auxiliary battery voltage fall below defined minimums.  Fault Mode has no exit and is therefore a termination mode.

This version of EV13 is designed to work specifically with a 6.6KW charger, 4.0 charging firmware and 4.0 charging hardware.  Traction pack and auxiliary battery topping off is optional once the controller has entered completion mode.  Additionally, this version of firmware is capable of leveling the traction pack after every nth charge complete cycle.

## Software Installation

Instructions for installing or updating software may be found in the Electric Vehicle Diagnostic and Display Software User Guide.

# Glossary

## Customer.h Data File

Variables that can be set by the user which are found in the Customer.h data file:

| AMP_HOURS | Amp Hour capacity of the battery pack based on a 15 amp hour discharge rate. |
| --- | --- |
| AMPHR_MAXLIMIT | Maximum cumulative amphours allowed to be returned to the battery pack during a charging cycle. |
| AMPS_SCALE | This is the scaling factor used to calibrate the current returned to the battery pack, measured by the charge controller and a calibrated clamp-on meter. |
| BATTERIES | Number of batteries placed in a series string. |
| BYPASS_TRIPPT1 | Flag used to signify whether or not to bypass the trip point established for Mode 1. |
| BYPASS_TRIPPT2 | Flag used to signify whether or not to bypass the trip point established for Mode 2. |
| BYPASS_TRIPPT3 | Flag used to signify whether or not to bypass the trip point established for Mode 3. |
| BYPASS_TRIPPT4 | Flag used to signify whether or not to bypass the trip point established for Mode 4. |
| CELLS | Number of cells per battery. |
| DO_TOP_OFFS | If set to 1, signifies a requirement to perform traction pack and auxiliary battery topping off if pack voltage is less than or equal to V_START_LEVEL or auxiliary battery voltage falls to 12.4 volts or less. |
| I_COR | Constant current mode error gain factor. |
| IMAX_MODE1 | Maximum Current Allowed, Mode 1. Range 0 to 25 Amps. (2) |
| IMAX_MODE2 | Maximum Current Allowed, Mode 2. Range 0 to 25 Amps. (2) |
| IMAX_MODE3 | Maximum Current Allowed, Mode 3. Range 0 to 25 Amps. (2) |
| IMAX_MODE4 | Maximum Current Allowed, Mode 4. Range 0 to 25 Amps. (2) |
| ITRIP1 | Current trip point for Mode 1. Range 0 to 25 Amps. (2) |
| ITRIP2 | Current trip point for Mode 2. Range 0 to 25 Amps. (2) |
| ITRIP3 | Current trip point for Mode 3. Range 0 to 25 Amps. (2) |
| ITRIP4 | Current trip point for Mode 4. Range 0 to 25 Amps. (2) |
| LEVEL_AFTER | Number of charge complete cycles before mandatory traction pack leveling will occur. |
| MON_AUX_BATT | If set to 1, signifies a requirement to monitor the auxiliary battery voltage limits. |
| MON_DOLPHIN | If set to 1, signifies a requirement to monitor the Dolphin pre-charge minus line for fault detection. |
| MON_TEMP_SENSE | If set to 1, signifies a requirement to monitor temperature sensors. |
| PUD | Power Up Delay. Range 0 to 59 Seconds. |
| STRINGS | Number of parallel strings within the battery pack. |
| T1_HOURS | Mode 1 time-out. Range 0 to 255 hours. (4) |
| T1_MIN | Mode 1 time-out. Range 0 to 59 minutes. (4) |
| T2_HOURS | Mode 2 time-out. Range 0 to 255 hours. (4) |
| T2_MIN | Mode 2 time-out. Range 0 to 59 minutes. (4) |
| T3_HOURS | Mode 3 time-out. Range 0 to 255 hours. (4) |
| T3_MIN | Mode 3 time-out. Range 0 to 59 minutes. (4) |
| T4_HOURS | Mode 4 time-out. Range 0 to 255 hours. (4) |
| T4_MIN | Mode 4 time-out. Range 0 to 59 minutes. (4) |
| TEMP_COMP_FLAG | If set to 1, signifies a requirement to perform temperature compensation. |
| TEMP_COMP_MV | Millivolts per cell per degree Celsius for temperature compensation. Range 0 to 999 where 999 is equal to 9.99mv. |
| TEMP_MAX_VOLTS | Maximum voltage allowed while operating with the temperature compensation enabled. Range 0 to 500 Volts (1). Normally set at 2.6 volts per cell * # of Batteries * Voltage Scaling Factor (i.e. .4867v/lsb). |
| TEMP_MIN_VOLTS | Minimum voltage allowed while operating with the temperature compensation enabled. Range 0 to 500 Volts (1). Normally set at 2.2 volts per cell * # of Batteries * Voltage Scaling Factor (i.e. .4867v/lsb). |
| TEMP_NOMINAL_C | Nominal temperature to be used only when performing temperature compensation calculations. |
| TEMPMAX | Defines the maximum temperature (in Kelvin) allowed to be detected by the temperature sensors. |
| TEMPMIN | Defines the minimum temperature (in Kelvin) allowed to be detected by the temperature sensors. |
| TMAX | Safety timer. Range 0 to 255 hours (Time Max.). (5) |
| TYPE_BATTERY | Code which represents the type of batteries installed on the vehicle. |
| V_COR | Constant voltage mode error gain factor. |
| V_START_LEVEL | Minimum voltage level before entering automatic leveling mode option. Range 0 to 500 Volts. (1) |
| V1 | Constant Voltage in Mode 1. Range 0 to 500 Volts. (1) |
| V2 | Constant Voltage in Mode 2. Range 0 to 500 Volts. (1) |
| V3 | Constant Voltage in Mode 3. Range 0 to 500 Volts. (1) |
| V4 | Constant Voltage in Mode 4. Range 0 to 500 Volts. (1) |
| VMAX | Battery Pack voltage lid - absolute. Range 0 to 500 Volts. (1) |
| VMIN | Under-voltage lockout. Used to detect broken sense wire situations or a damaged battery pack. Range 0 to 500 Volts (1) . For Lead Acid, normally set at 1.66 volts per cell. |
| VOLTS_SCALE | This is the scaling factor used to calibrate the battery pack voltage measured by the charge controller and a calibrated meter. |

## Sensed Inputs

| DISC1 | Discrete input #1 - opto diode on = true. Presently not used. |
| --- | --- |
| DISC2 | Discrete input #2 - opto diode on = true. Presently not used. |
| DISC3 | Discrete input #3 - opto diode on = true. Presently not used. |
| ISENSE | Range 0 to 25 Amps = 8 bits. (2) |
| TSENSE1 | Range -30 to +75°C equals = 8 bits. |
| TSENSE2 | Range -30 to +75°C equals = 8 bits. |
| V_AUXBATTERY | Range 0 to 25 Volts = 8 bits. |
| V_CH_PORT | Range 0 to 25 Volts = 8 bits. |
| V_PRECHARGE | Range 0 to 25 Volts = 8 bits. |
| VSENSE | Range 0 to 500 Volts = 8 bits (1). |

## Outputs

| J1850 | Serial communication channel. Messages per J1850 specification. |
| --- | --- |
| DISC-OUT | Discrete output, used to switch on/off external functions (i.e. ventilation fans) by using a relay within the charge controller. Enabled at detection of connection present and disabled when Completion or Fault Mode is encountered. |
| RS232 | Charge Controller data monitoring and program loading port. |

## Notes

| Note 1 | The present 6.6kw SCM is limited to 405 Volts output. |
| --- | --- |
| Note 2 | The present 6.6kw SCM is limited to 20 Amps output. |
| Note 3 | Variable settings are not necessarily in the units as shown above. See program listing for details. |
| Note 4 | Mode time-outs can be set in hours and / or minutes. For example time for Mode 1 is actually set using T1_HOURS and T1_MIN. |
| Note 5 | For TMAX to operate properly, it must be set to at least the sum of T1_HOURS, T1_MIN, T2_HOURS, T2_MIN, T3_HOURS, T3_MIN, T4_HOURS, and T4_MIN. Setting TMAX less than the sum of these settings may cause a premature exit from the charging process due to exceeding TMAX. |
| Note 6 | See Figure 2, found later in this document, for a typical charging profile. |

# Modes of Operation

There are presently seven different modes of operation.  Briefly, Mode 0 (zero) is a power up, self test mode; Modes 1 and 2 are Constant Voltage modes; Mode 3 is a Constant Voltage mode used to level the traction pack; Mode 4 is a Constant Voltage mode used to recharge the auxiliary battery; Mode 254 is the Completion Mode; and Mode 255 is the Fault Mode.  The operation of Mode 1 and 2 is sequential, ending with Completion Mode.  Mode 3 ends with Completion Mode.  Mode 4 ends with Completion Mode.  Fault Mode may be entered at any time.  Exit from Completion Mode may occur if the DO_LEVELING flag has been set and the required parameters have been met.  Exit from Fault Mode is not allowed.  Modes 1 through 4 may be terminated when a fault condition is detected, elapsed time has met or exceeded defined limits, or when the trip point defined for the specific mode has been met.  Safety checks are continuously performed during the charging process.  Should a safety violation be detected, Fault Mode is immediately entered.  Detailed discussion of each mode is provided below.

## Mode 0 (Power Up Delay)

Upon power up, the Charge Controller performs all necessary initialization and self test routines.  It then performs a power up delay (PUD), based on the customers preset value, where no vehicle J1850 communication messages are broadcast.  The Charge Command is set preset to zero(0).  After completing the specified delay, transition to Mode 1, or Fault Mode if appropriate, occurs.  During Mode 0, no data is transmitted to the RS232 port.

## Mode 2 (Constant Voltage #1)

Control mode is constant voltage.  The Charge Command is adjusted to maintain a constant voltage based on the setting of Mode 1 parameter V1.  Mode 1 is terminated and operation goes to Mode 2 if ISENSE £ ITRIP1, or if the elapsed time in Mode 1 equals or exceeds the sum of parameters (T1_HOURS + T1_MIN). If the BYPASS_TRIPPT1 was set (in the customer.h file), operation of Mode 1 will continue, without regard to the ITRIP1 trip point, until the elapsed time in Mode 1 equals or exceeds the sum of parameters (T1_HOURS + T1_MIN). Safety checks are continuously performed while in this mode of operation.  If a fault was detected during this mode of operation, transition will be to Fault Mode.

## Mode 2 (Constant Voltage #2)

Control mode is constant voltage.  The Charge Command is adjusted to maintain a constant voltage based on the setting of Mode 2 parameter V2.  Mode 2 is terminated and operation goes to Completion Mode if ISENSE £ ITRIP2, or if the elapsed time in Mode 2 equals or exceeds the sum of parameters (T2_HOURS + T2_MIN). If the BYPASS_TRIPPT2 was set (in the customer.h file), operation of Mode 2 will continue, without regard to the ITRIP2 trip point, until the elapsed time in Mode 2 equals or exceeds the sum of parameters (T2_HOURS + T2_MIN). Safety checks are continuously performed while in this mode of operation.  If a fault was detected during this mode of operation, transition will be to Fault Mode.

Upon completion of this mode, a comparison is made of the number of successful charge complete cycles and the LEVEL_AFTER constant.  If the number of charge complete cycles equals the LEVEL_AFTER constant, traction pack leveling (Mode 3) is entered.

This mode may be re-entered after Completion Mode has been entered, and only if the DO_LEVELING flag has been set, and the traction pack voltage has been detected at V_START_LEVEL volts or less. A check of the traction pack is performed once per hour, based on the Completion Mode clock.  If the traction pack voltage is found to be low during this check, Mode 2 is once again entered and normal operation of Mode 2 will occur as defined above.

## Mode 3 (Traction Pack Leveling - Constant Voltage)

This mode may only be entered when the number of successfully completed charging cycles equals the number defined in LEVEL_AFTER constant, specified in the customer.h file. Control mode is constant voltage.  The Charge Command is adjusted to maintain a constant voltage based on the setting of Mode 3 parameter V3.  Mode 3 is terminated and operation goes to Completion Mode if ISENSE £ ITRIP3, or if the elapsed time in Mode 3 equals or exceeds the sum of parameters (T3_HOURS + T3_MIN).  If the BYPASS_TRIPPT3  was set (in the customer.h file) operation of Mode 3 will continue, without regard to the ITRIP3 trip point, until the elapsed time in Mode 3 equals or exceeds the sum of parameters (T3_HOURS + T3_MIN). Safety checks are continuously performed while in this mode of operation.  If a fault was detected during this mode of operation, transition will be to Fault Mode.  The DO_LEVELING flag has no bearing on the execution of this mode.

## Mode 4 (Auxiliary Battery Charging - Constant Voltage)

This mode may only be entered from Completion Mode, and only if the DO_LEVELING flag has been set, and the auxiliary battery voltage has been detected at less than approximately 12.4 volts. Control mode is constant voltage.  The Charge Command is adjusted to maintain a constant voltage based on the setting of Mode 4 parameter V4.  Mode 4 is terminated and operation goes to Completion Mode if ISENSE £ ITRIP4, or if the elapsed time in Mode 4 equals or exceeds the sum of parameters (T4_HOURS + T4_MIN). If the BYPASS_TRIPPT4  was set (in the customer.h file) operation of Mode 4 will continue, without regard to the ITRIP4 trip point, until the elapsed time in Mode 4 equals or exceeds the sum of parameters (T4_HOURS + T4_MIN). Safety checks are continuously performed while in this mode of operation.  If a fault was detected during this mode of operation, transition will be to Fault Mode.

## Mode 254 (Completion Mode)

Mode 254, or Completion Mode, is an idle state.  The Charge Command is set to zero (0) and Charge Complete is indicated on the Standard Charge Module (SCM).  The State Of Charge is set to 100%.  Vehicle J1850 communication messages are then limited in scope.  If the traction pack and/or auxiliary battery leveling (DO_LEVELING) option has been enabled, and the prescribed minimum voltages are reached, re-entry into Mode 2 or entry into Mode 3 will occur.

The auxiliary battery voltage is monitored at approximately 10 Hz while in Completion Mode.  Should the voltage fall below V_AUX_BATT_MON (defaulted to approximately 12.4vdc), the Dolphin system will be enabled (PA7 turned on), a 5 second delay will be entered, charging will start in Mode 4, lasting for the duration specified in Mode 4's parameters.  Upon completion of the charging cycle, charging will terminate, and completion mode will be re-entered.  Auxiliary battery voltage monitoring will then occur as before.

The traction pack voltage will be monitored once per hour. Should the voltage fall below V_START_LEVEL as defined in the customer.h file, the Dolphin system will be enabled (PA7 turned on), a 5 second delay will be entered, charging will start in Mode 3, utilizing normal charge algorithms, trip, and set points. Upon completion of the charging cycle, charging will terminate, and completion mode will be re-entered.  Traction pack voltage monitoring will then occur as before, once per hour.

## Mode 255 (Fault Mode - Safety - Applicable During All Modes)

Mode 255, or Fault Mode, may be entered from any mode.  Upon entry into Fault Mode, charging is immediately terminated, the Charge Command is set to zero (0), and a Check Vehicle message is displayed on the SCM.  Fault Mode is considered to be an idle state and as a safety feature, no transition out of Fault Mode is allowed.  Reasons for a fault include:

VSENSE > VMAX

VSENSE < VMIN

TSENSE1 > TEMPMAX

TSENSE2 > TEMPMAX

Time since power up (safety timer) > TMAX

Charge Port OVERTEMP discrete being set

Auxiliary Battery voltage > 14.3v

Auxiliary Battery voltage < 11.2v

Dolphin unit Lid Open (if applicable)

Dolphin FET is Off (if applicable)

Vehicle Key being turned on

AmpHrs applied > AMPHR_MAXLIMIT

Any other fault causing charging to be terminated.

# Filters

The EV13xxx program supplements the hardware filtering in the Charge Controller by using an Infinite Impulse Response (IIR) software filter for the current, temperature and voltage measurements.  The mechanization of the IIR Filter is complex and should not require modification.  For information purposes, the corner frequency of the IIR Filter is set at 10 Hz.  This affords good noise filtering without a significant phase shift at the update frequencies.

# Stability Considerations

Stability is of great concern as not all parameters of the various systems are known and modeling has only been associated with first order affects.  The program listing contains a very good description of how to set the V_COR and I_COR error gain factors and how they affect system stability.  In general, the higher the stability variables are set, the faster the error correction is initiated.  However, there is also a greater risk of instability.  It is suggested to set V_COR and I_COR as low as practical for acceptable charging performance to maintain a good stability margin.  For further information, please see the Magne-Charge Control Theory of Operation and the program listing.

# Temperature Compensation

Modes 1 through 4 support the optional use of temperature compensation.  To enable this option, a label called TEMP_COMP_FLAG must be set to a value of one (1).  If temperature compensation is not desired, the value for this label should be set to zero (0).

Temperature compensation utilizes multiple constants which are found within the Customer.h data file.  The constants control the performance of the temperature compensation algorithm and, if set improperly, could cause improper battery charging.  The following constants are used:

TEMP_MAX_VOLTS

TEMP_MIN_VOLTS

TEMP_COMP_MV

TEMP_NOMINAL_C

Note:  	Temperature compensation adjusts the variables when battery temperatures vary from 25 degrees centigrade.  Presently set-up for lead acid battery only.  NiCad, etc. will have a different routine and constants.

Refer to section 12 of this document to determine the values to preset in the above constants.

# State of Charge

The battery pack state of charge indication is an approximation calculated in the charging software.  Since the amount of energy utilized by the vehicle while being driven is not tracked by the charge controller, only an approximation can be made concerning a battery pack’s actual state of charge.  The final product of the state of charge calculation is sent to the SCM (charger) and displayed on the front panel.  For this calculation to be fairly accurate, constants within the Customer.h data file must be preset.  These constants are as follows:

AMP_HOURS

BATTERIES

CELLS

STRINGS

Some factors that will affect the displayed state of charge, besides the above constants, are as follows:

Present state of charge on the battery pack.

Time elapsed since the last charge was applied.

Time elapsed since the last draw down on the battery pack (i.e. vehicle use).

Note:  	The state of charge calculation is presently set-up for a lead acid battery only.  NiCad, etc. will have a different routine and constants.

Refer to section 12 of this document to determine the values to preset in the above constants.

# Brown-Out Conditions

Inclement weather and peak demands on utility resources may present a condition where facility input power for the SCM becomes unstable.  During these periods, the SCM may enter a power-loss or experience a brown-out condition.  When this occurs, the charge controller will inhibit updates to the charge command (within 3 seconds) and a brown-out delay loop is automatically entered.  The main purpose for the delay loop is to protect the hardware (vehicle and SCM) during and immediately after the power-loss or brown-out.  This loop is maintained until either power is fully restored or a specified time has elapsed.  Should power be restored, another brief delay loop (10 seconds) is entered to allow the SCM to stabilize.  Upon completion of the 10 second delay loop, the charge command is free to update.  Should power not be restored within the specified time limit, presently set at 30 seconds,  Fault Mode is entered.

The maximum allowed duration of a brown-out condition is defaulted to 30 seconds.  Should the brown-out last more than thirty 30 seconds, the charge controller will enter Fault Mode.  Requests to change the default time limit should be made to the Point of Contact listed at the end of this document.

# Monitoring (Via RS232 Port)

The RS232 Port is a critical aid to troubleshooting and understanding system operation.  The RS232 Port is fed a continuous stream (asynchronous) of information regarding system operation.  This data can be monitored and recorded utilizing the accompanying Windows based diagnostic program.  The information sent out the RS232 Port is updated approximately three times per second and contains:

Software program name

Software revision date

Raw and filtered voltage

Raw and filtered current

Raw and filtered temperatures

Present charging mode

Present mode clock

System (watchdog) time since power up

J1850 messages

Mode set and trip values

Many other variables and settings.

It is not mandatory to monitor the RS232 Port for correct charger operation.  Refer to the Electric Vehicle Diagnostic and Display Software User Guide for further information.

# Auxiliary Battery Protection

The auxiliary battery is used to provide power to the charge controller.  During a normal charging operation, the auxiliary battery is recharged by the Dolphin unit.  Should this recharging circuit fail, the auxiliary battery may eventually deplete it’s power to an unusable level.  Likewise, should the recharging system fail and continue to charge the auxiliary battery beyond 100% state of charge, battery damage may occur.

To prevent either of the above conditions from occurring, it is possible to monitor the auxiliary battery voltage level and have the charge controller enter Fault Mode should a fault (overvolt or undervolt) occur.  Within the Customer.h file is a flag called MON_AUX_BATT which will provide the desired protection.  By setting the flag to a 1, protection is enabled and faults with the auxiliary battery will be detected.  If the flag is set to 0, no fault detection or protection is provided for the auxiliary battery.

It is highly recommended to enable this feature for safety and equipment protection.

# Dolphin Protection

The Dolphin unit, if installed, is the heart of the EV installation.  To prevent damage to the unit, additional safety protection has been provided if desired.  Should the Dolphin unit experience a malfunction during the charging process, the Dolphin will trip off-line and charging will terminate.  If charging does not terminate when a malfunction occurs, the pre-charge circuit within the Dolphin unit is subject to damage.

To prevent damage from occurring, it is possible to monitor the charging status of the Dolphin unit and have the charge controller enter Fault Mode should a fault occur.  Within the Customer.h file is a flag called MON_DOLPHIN which will provide the desired protection.  By setting the flag to a 1, protection is enabled and faults with the Dolphin unit will be detected.  If the flag is set to 0, no fault detection or protection is provided for the Dolphin unit and damage to the pre-charge circuit could occur should the Dolphin malfunction.

It is highly recommended to enable this feature for safety and equipment protection.

# Customer.h Equates File

This section describes the contents of the Customer.h file found on the distribution disk.  The constants and variables listed below are provided by the customer and used to develop the appropriate charging algorithm.  A brief description is provided for each entry in the Customer.h file, followed by a valid range, default value and a copy of the existing entry.  When making changes to the formula entries below, only change the integer value which is underlined.  No spaces are permitted between values in the formulas.  Do not remove or change the curly brackets { } contained in any of the formulas.  They are required for the assembler.  The integer items within the curly brackets are underlined in the examples within this section.  Refer to the Cross-32 Meta-Assembler for further information concerning field entries.

## Calibration Settings

These constants are the scaling factors used to formulate set and trip points, multiplied by either VOLTS_MULTIPLY or AMPS_MULTIPLY.  To determine the proper settings, first assemble the program and then start the charging process.  Next, determine the voltage and current readings from the diagnostic program.  Then take actual measurements for both voltage and current.  The following formula may then be used to determine the new calibration settings:

Diagnostic Displayed Reading

New Cal Setting =   ¾¾¾¾¾¾¾¾¾¾¾¾¾ 	*	Present Scaling Factor

Actual Measured Reading

Default Setting for Voltage = 4867  				;   .4867 * 10000

Default Setting for Current = 1050  				; 10.50   * 100

VOLTS_SCALE:	EQU		4867						;   .4867 * 10000

AMPS_SCALE:		EQU		1050						; 10.50   * 100

## Battery Information

There are multiple entries required for the installed battery pack.

### Type of Battery

Select from the listing below the type of battery installed in the vehicle.  The battery type is presently used for display back to the user via the Windows based diagnostic utility program.  It has no affect on calculations performed in the charging algorithms.

If the battery type is not listed below, use type zero (0) and notify the point of contact, listed at the end of this document.  Additional battery types can be added to the listing.

| 0 | Type of Battery Unknown or Not Listed |
| --- | --- |
| 1 | Gates Sealed Lead Acid G12V38AH |
| 2 | Gates Sealed Lead Acid G12V26AH |
| 3 | GNB Sealed Lead Acid Upsolyte |
| 4 | GNB Sealed Lead Acid Evolyte |
| 5 | GNB Sealed Lead Acid Champion |
| 6 | Sonnenshein Sealed Lead Acid |
| 7 | Trojan TEB225 |
| 8 | Genesis Sealed Lead Acid G12V38AH10C |
| 9 | Electrosource 12N95 |
| 10 | Ovonic OVC12 Nickel Metal Hydride |

Valid Range		= 1 - 255

Default Value	= 0

TYPE_BATTERY:		EQU		0				;Battery Type defined as Unknown

### Battery (BATTERIES)

This defines the total number of batteries placed in each series string for this vehicle.

Valid Range		= 1 to 255 batteries

Default Value	= 26

BATTERIES:		EQU		26			;Number of batteries in series

### Strings (STRINGS)

Strings defines the number of battery strings found within the installed battery pack.

Valid Range		= 1 to 255 strings

Default Value	= 1

STRINGS:			EQU		1			;Number of battery strings in the battery pack.

### Cells (CELLS)

Cells defines the total number of cells per battery.

Valid Range		= 1 to 255 cells

Default Value	= 6

CELLS:				EQU		6			;Number of cells per battery

### Ampere Hours (AMP_HOURS)

This is the ampere hour capacity of the battery based on a 15 ampere discharge rate.

Valid Range		= 1 to 255 ampere hours

Default Value	= 60

AMP_HOURS:	EQU		60				;Amp hour capacity of Battery Pack

### Battery Maximum Voltage (VMAX)

The maximum battery pack voltage allowed for this vehicle.  If voltage exceeds this level, charging will terminate and Fault Mode will be entered.  It should be noted that presently, the Dolphin unit is set for an internal VMAX of 400 volts.  Setting the charge controller to a value greater than that of the Dolphin unit will only result in the Dolphin unit shutting down when it’s VMAX limit is reached.  VMAX should be set above TEMP_MAX_VOLTS.  See TEMP_MAX_VOLTS.

Valid Range		= 0 to 500 volts  (1)

Default Value	= 400 volts  (1)

Normal Value	= (2.8 volts per cell) (number of cells) (number of batteries)

VMAX:				EQU		{400*VOLTS_SCALE}/VOLTS_MULTIPLY

### Battery Minimum Voltage (VMIN)

The minimum battery pack voltage allowed for this vehicle.  If voltage is detected below this level, charging will be inhibited and Fault Mode will be entered.  Note: VMIN should be set below TEMP_MIN_VOLTS.  See TEMP_MIN_VOLTS.

Valid Range		= 0 to 500 volts  (1)

Default Value	= 260 volts

Normal Value	= (1.66 volts per cell) (number of cells) (number of batteries)

VMIN:				EQU		{260*VOLTS_SCALE}/VOLTS_MULTIPLY

## Power Up Delay (PUD).

This is the amount of time delay, after connection present is detected, until the charging process begins.  Used to initiate a contactor, ventilation fans, auxiliary supplies, etc., prior to safety fault checking.

Valid Range		=	0 to 59 seconds

Default Value	=	10 seconds

Normal Value	=	10 Seconds

PUD:				EQU		10					; Power up delay

## Time Maximum (TMAX)

This is the maximum cumulative charging time allowed, in hours, for all modes combined.  If the system clock exceeds this time limit, the charge controller will enter Fault Mode.  The primary function of this constant is a safety function.

Valid Range		=	0 to 255 hours

Default Value	=	24 hours

Normal Value	=	24 hours

TMAX:				EQU		24					;Total Maximum system charging time

## Mode Parameter Settings

The following section describes the settings required for each mode of operation.  Be sure the proper calculations are used and the formulas are not modified.  Most parameters are defaulted to zero (0) because it is unknown as to the customers requirements.

### Mode 1 - Constant Voltage #1

#### Mode 1 Voltage Set Point (V1)

This is the charging voltage that will be maintained while in Mode 1 based on 25 degrees centigrade battery temperature.

Valid Range		= 0 to 500 (1) volts dc

Default Value	= 0 volts

V1:					EQU		{0*VOLTS_SCALE}/VOLTS_MULTIPLY

#### Mode 1 Current Trip Point (ITRIP1)

This is the amperage that will cause the charge controller to trip from Mode 1 into Mode 2.  Entries must be integer numbers.  Multiply the required entry by a factor of 10 before entering.  For instance, if the desired amperage is 15, an entry of 150 (15 * 10) is required.  To set 12.7 amps, enter 127 (12.7 * 10).

Valid Range		= 0 to 25 (2)  amps

Default Value	= 0 amps

ITRIP1:				EQU		{{0*CURRENT_SCALE}/{AMPS_MULTIPLY*TEN}}

#### Mode 1 Maximum Current Allowed (IMAX_MODE1)

This is the maximum amount of current which may be applied while charging in Mode 1. This is used as a safety feature to prevent the charger from applying excess current when deficiencies may exist with the battery pack. Entries must be integer numbers.  Multiply the required entry by a factor of 10 before entering.  For instance, if the desired amperage is 15, an entry of 150 (15 * 10) is required.  To set 12.7 amps, enter 127 (12.7 * 10).

Note:  This constant must contain a value greater than zero for charging to occur.

Valid Range 	= 0 to 25 (2)  amps

Default Value	= 0 Amps

Normal Value	= 20 Amps

IMAX_MODE1:		EQU		{{0*CURRENT_SCALE}/{AMPS_MULTIPLY*TEN}}

#### Bypass Mode 1 Trip Point (BYPASS_TRIPPT1)

The bypass trip point 1 flag indicates the trip point for Mode 1 is to be ignored.  This will then cause Mode 1 to operate until the time settings in (T1_HOURS + T1_MIN) has elapsed.  The only other possible way to trip from this mode when the bypass flag is set would be if a fault was detected.

Valid Range 		=	0 = Don't bypass Mode 1 trip point.

1 = Do bypass Mode 1 trip point.

Default Value	=	0 = Don't bypass Mode 1 trip point.

BYPASS_TRIPPT1:			EQU     0 		;Don't bypass Mode 1 trip point.

#### Mode 1 Time in Hours and Minutes (T1_HOURS & T1_MIN)

This is the maximum amount of charging time allowed, in hours and minutes, for Mode 1.  Note:  Setting T1_HOURS and T1_MIN to zero will cause the system to skip Mode 1 and proceed directly to Mode 2.

Valid Range		= 0 to 24 hours

Valid Range		= 0 to 59 minutes

Default Value	= 0 hours

Default Value	= 0 minutes

T1_HOURS:		EQU		0			;Mode 1 time-out in hours

T1_MIN:			EQU		0			;Mode 1 time-out in minutes

### Mode 2 - Constant Voltage #2

#### Mode 2 Voltage Set Point (V2)

This is the charging voltage that will be maintained while in Mode 2 based on 25 degrees centigrade battery temperature.

Valid Range		= 0 to 500 (1) volts dc

Default Value	= 0 volts

V2:					EQU		{0*VOLTS_SCALE}/VOLTS_MULTIPLY

#### Mode 2 Current Trip Point (ITRIP2)

This is the amperage that will cause the charge controller to trip from Mode 2 into Completion Mode.  Entries must be integer numbers.  Multiply the required entry by a factor of 10 before entering.  For instance, if the desired amperage is 15, an entry of 150 (15 * 10) is required.  To set 12.7 amps, enter 127 (12.7 * 10).

Valid Range		= 0 to 25 (2)  amps

Default Value	= 0 amps

ITRIP2:				EQU		{{0*CURRENT_SCALE}/{AMPS_MULTIPLY*TEN}}

#### Mode 2 Maximum Current Allowed (IMAX_MODE2)

This is the maximum amount of current which may be applied while charging in Mode 2.  This is used as a safety feature to prevent the charger from applying excess current when deficiencies may exist with the battery pack. Entries must be integer numbers.  Multiply the required entry by a factor of 10 before entering.  For instance, if the desired amperage is 15, an entry of 150 (15 * 10) is required.  To set 12.7 amps, enter 127 (12.7 * 10).

Note:  This constant must contain a value greater than zero for charging to occur.

Valid Range 	= 0 to 25 (2)  amps

Default Value	= 0 Amps

Normal Value	= Value used for I1 setting from Mode 1, if using Mode 1.  Set to 20 amps if not using Mode 1.

IMAX_MODE2:		EQU		{{0*CURRENT_SCALE}/{AMPS_MULTIPLY*TEN}}

#### Bypass Mode 2 Trip Point (BYPASS_TRIPPT2)

The bypass trip point 2 flag indicates the trip point for Mode 2 is to be ignored.  This will then cause Mode 2 to operate until the time settings in (T2_HOURS + T2_MIN) has elapsed.  The only other possible way to trip from this mode when the bypass flag is set would be if a fault was detected.

Valid Range 		=	0 = Don't bypass Mode 2 trip point.

1 = Do bypass Mode 2 trip point.

Default Value	=	0 = Don't bypass Mode 2 trip point.

BYPASS_TRIPPT2:			EQU     0 		;Don't bypass Mode 2 trip point.

#### Mode 2 Time in Hours and Minutes (T2_HOURS & T2_MIN)

This is the maximum amount of charging time allowed, in hours and minutes, for Mode 2.  Note:  Setting T2_HOURS and T2_MIN to zero will cause the system to skip Mode 2 and proceed directly to Completion Mode.

Valid Range		= 0 to 24 hours

Valid Range		= 0 to 59 minutes

Default Value	= 0 hours

Default Value	= 0 minutes

T2_HOURS:		EQU		0			;Mode 2 time-out in hours

T2_MIN:			EQU		0			;Mode 2 time-out in minutes

### Mode 3 - Traction Pack Leveling - Constant Voltage

#### Mode 3 Voltage Set Point (V3)

This is the charging voltage that will be maintained while in Mode 3 based on 25 degrees centigrade battery temperature.

Valid Range		= 0 to 500 (1) volts dc

Default Value	= 0 volts

V3:					EQU		{0*VOLTS_SCALE}/VOLTS_MULTIPLY

#### Mode 3 Current Trip Point (ITRIP3)

This is the amperage that will cause the charge controller to trip from Mode 3 into Completion Mode.  Entries must be integer numbers.  Multiply the required entry by a factor of 10 before entering.  For instance, if the desired amperage is 15, an entry of 150 (15 * 10) is required.  To set 12.7 amps, enter 127 (12.7 * 10).

Valid Range		= 0 to 25 (2)  amps

Default Value	= 0 amps

ITRIP3:				EQU		{{0*CURRENT_SCALE}/{AMPS_MULTIPLY*TEN}}

#### Mode 3 Maximum Current Allowed (IMAX_MODE3)

This is the maximum amount of current which may be applied while charging in Mode 3. This is used as a safety feature to prevent the charger from applying excess current when deficiencies may exist with the battery pack. Entries must be integer numbers.  Multiply the required entry by a factor of 10 before entering.  For instance, if the desired amperage is 15, an entry of 150 (15 * 10) is required.  To set 12.7 amps, enter 127 (12.7 * 10).

Note:  This constant must contain a value greater than zero for charging to occur.

Valid Range 	= 0 to 25 (2)  amps

Default Value	= 0 Amps

Normal Value	= 20 Amps

IMAX_MODE3:		EQU		{{0*CURRENT_SCALE}/{AMPS_MULTIPLY*TEN}}

#### Bypass Mode 3 Trip Point (BYPASS_TRIPPT3)

The bypass trip point 3 flag indicates the trip point for Mode 3 is to be ignored.  This will then cause Mode 3 to operate until the time settings in (T3_HOURS + T3_MIN) has elapsed.  The only other possible way to trip from this mode when the bypass flag is set would be if a fault was detected.

Valid Range 		=	0 = Don't bypass Mode 1 trip point.

1 = Do bypass Mode 1 trip point.

Default Value	=	0 = Don't bypass Mode 1 trip point.

BYPASS_TRIPPT3:			EQU     0 		;Don't bypass Mode 3 trip point.

#### Mode 3 Time in Hours and Minutes (T3_HOURS & T3_MIN)

This is the maximum amount of charging time allowed, in hours and minutes, for Mode 3.  Note:  Setting T3_HOURS and T3_MIN to zero will cause the system to skip Mode 3 and proceed directly to Completion Mode.

Valid Range		= 0 to 24 hours

Valid Range		= 0 to 59 minutes

Default Value	= 0 hours

Default Value	= 0 minutes

T3_HOURS:		EQU		0			;Mode 3 time-out in hours

T3_MIN:			EQU		0			;Mode 3 time-out in minutes

### Mode 4 - Auxiliary Battery Charging - Constant Voltage

#### Mode 4 Voltage Set Point (V4)

This is the charging voltage that will be maintained while in Mode 4 based on 25 degrees centigrade battery temperature.

Valid Range		= 0 to 500 (1) volts dc

Default Value	= 0 volts

V4:					EQU		{0*VOLTS_SCALE}/VOLTS_MULTIPLY

#### Mode 4 Current Trip Point (ITRIP4)

This is the amperage that will cause the charge controller to trip from Mode 4 into Completion Mode.  Entries must be integer numbers.  Multiply the required entry by a factor of 10 before entering.  For instance, if the desired amperage is 15, an entry of 150 (15 * 10) is required.  To set 12.7 amps, enter 127 (12.7 * 10).

Valid Range		= 0 to 25 (2)  amps

Default Value	= 0 amps

ITRIP4:				EQU		{{0*CURRENT_SCALE}/{AMPS_MULTIPLY*TEN}}

#### Mode 4 Maximum Current Allowed (IMAX_MODE4)

This is the maximum amount of current which may be applied while charging in Mode 4. This is used as a safety feature to prevent the charger from applying excess current when deficiencies may exist with the battery pack. Entries must be integer numbers.  Multiply the required entry by a factor of 10 before entering.  For instance, if the desired amperage is 15, an entry of 150 (15 * 10) is required.  To set 12.7 amps, enter 127 (12.7 * 10).

Note:  This constant must contain a value greater than zero for charging to occur.

Valid Range 	= 0 to 25 (2)  amps

Default Value	= 0 Amps

Normal Value	= 20 Amps

IMAX_MODE4:		EQU		{{0*CURRENT_SCALE}/{AMPS_MULTIPLY*TEN}}

#### Bypass Mode 4 Trip Point (BYPASS_TRIPPT4)

The bypass trip point 4 flag indicates the trip point for Mode 4 is to be ignored.  This will then cause Mode 4 to operate until the time settings in (T4_HOURS + T4_MIN) has elapsed.  The only other possible way to trip from this mode when the bypass flag is set would be if a fault was detected.

Valid Range 		=	0 = Don't bypass Mode 1 trip point.

1 = Do bypass Mode 1 trip point.

Default Value	=	0 = Don't bypass Mode 1 trip point.

BYPASS_TRIPPT4:			EQU     0 		;Don't bypass Mode 4 trip point.

#### Mode 4 Time in Hours and Minutes (T4_HOURS & T4_MIN)

This is the maximum amount of charging time allowed, in hours and minutes, for Mode 4.  Note:  Setting T4_HOURS and T4_MIN to zero will cause the system to skip Mode 4 and proceed directly to Completion Mode.

Valid Range		= 0 to 24 hours

Valid Range		= 0 to 59 minutes

Default Value	= 0 hours

Default Value	= 0 minutes

T4_HOURS:		EQU		0			;Mode 4 time-out in hours

T4_MIN:			EQU		0			;Mode 4 time-out in minutes

## Temperature

### Temperature Sensor Monitoring (MON_TEMP_SENSE)

Temperature sensors are normally installed in the battery pack area to assist in thermal management of the batteries.  If this constant is set to a one (1), the temperature sensors will be enabled.  If set to a zero (0), no monitoring of the temperature sensors will occur.  Set this flag to 1 only if the additional hardware for temperature sensors has been installed in the vehicle.  Setting this flag to a 1 without temperature sensors installed may cause damage to the battery pack as the algorithms will be processed incorrectly.

Valid Range		=	0	=	Don't perform Temperature Sensor monitoring.

=	1	=	Do perform Temperature Sensor monitoring.

Default Value	=	0	=	Don't perform Temperature Sensor monitoring.

MON_TEMP_SENSE: 	EQU		0				;Don't Monitor Temperature Sensors

### Temperature Maximum (TEMPMAX).

Temperature Maximum defines the maximum temperature allowed before reporting an overheat condition with the battery pack.  The temperature sensors are capable of reporting temperatures from 243 degrees Kelvin (-30 degrees Celsius) to 348 degrees Kelvin (+75 degrees Celsius). If temperature exceeds this value, Fault Mode is entered.

In the following formula, 243 equals the minimum temperature in Kelvin which can be read by the temperature sensors; 50 equals .5 times 100 for rounding the result to an integer; 100 equals the offset used on LSB_PER_KELVIN to make it an integer.  The constant LSB_PER_KELVIN is defined in the EQUATES.H file.

Valid Range		=	243 to 348 degrees Kelvin (-30°c to +75°c)

Normal Value	=	348 degrees Kelvin (+75°c)

Default Value	=	348 degrees Kelvin (+75°c)

TEMPMAX:	EQU	{{{{348*LSB_PER_KELVIN}-{243*LSB_PER_KELVIN}}+50}/100}

### Temperature Minimum (TEMPMIN).

TEMPMIN defines the minimum temperature (in Kelvin) allowed to be detected by the temperature sensors.  If temperature drops below this value, Fault Mode is entered.

In the following formula, 243 equals the minimum temperature in Kelvin which can be read by the temperature sensors; 50 equals .5 times 100 for rounding the result to an integer; 100 equals the offset used on LSB_PER_KELVIN to make it an integer.  The constant LSB_PER_KELVIN is defined in the EQUATES.H file.

Valid Range		=	243 to 348 degrees Kelvin (-30°c to +75°c)

Normal Value	=	253 degrees Kelvin(-20°c)

Default Value	=	253 degrees Kelvin(-20°c)

TEMPMIN: EQU  {{{{253*LSB_PER_KELVIN}-{243*LSB_PER_KELVIN}}-50}/100}

### Temperature Compensation Flag (TEMP_COMP_FLAG)

This constant should be set to a 1 (one) if temperature compensation is desired.  If temperature compensation is not desired, set this constant to a 0 (zero).

Note:  It is important to have the flag set to zero (no temperature compensation performed) if the vehicle configuration does not include temperature sensors.  Failure to set this flag to zero may cause erratic charging voltages as the sensors will default to approximately -23 degrees Celsius.  This would then cause all voltage set points and trip points to be increased.

Valid Range		= 0	Don't perform temperature compensation

= 1	Perform temperature compensation

Default Value	= 0	Don't perform temperature compensation

TEMP_COMP_FLAG: 	EQU		0		;0 = No Temperature Compensation

### Temperature Compensation Maximum Voltage (TEMP_MAX_VOLTS)

Define the maximum voltage limit for the temperature compensation routine, if enabled (see TEMP_COMP_FLAG). Applied charging voltage will not exceed TEMP_MAX_VOLTS.  See VMAX.

Valid Range		= 0 to 500  (1)  volts dc

Default Value	= 398 volts

Normal Value	= (2.6 volts per cell) (number of cells) (number of batteries)

TEMP_MAX_VOLTS:	EQU		{398*VOLTS_SCALE}/VOLTS_MULTIPLY

### Temperature Compensation Minimum Voltage (TEMP_MIN_VOLTS)

Define the minimum voltage limit for the temperature compensation routine, if enabled (see TEMP_COMP_FLAG).  Applied charging voltage should not fall below TEMP_MIN_VOLTS.  See VMIN.

Valid Range		= 0 to 500  (1)  volts dc

Default Value	= 343 volts

Normal Value	= (2.2 volts per cell) (number of cells) (number of batteries)

TEMP_MIN_VOLTS:		EQU		{343*VOLTS_SCALE}/VOLTS_MULTIPLY

### Temperature Compensation Millivolts (TEMP_COMP_MV)

Define the number of Millivolts required for temperature compensation adjustment per cell, per degree Celsius.  When entering values, multiply the actual decimal value by 100,000 to provide an integer number.  This is mandatory.

Valid Range		=	0 to 999 where 999 equals 9.99mv per cell per degree Celsius.

Default Value	=	444	(.00444 volts or 4.44mv)

TEMP_COMP_MV:		EQU		444

### Temperature Nominal Degrees Celsius (TEMP_NOMINAL_C)

Define the nominal temperature, based on the battery type, which should be used when calculating the offset voltage for temperature compensation for this EV application.  This information is only for use in temperature compensation.

Valid Range		= 0 to 60 where 60 equals 60 degrees Celsius

Default Value	= 25 degrees Celsius

Normal Value	= 25 degrees Celsius

TEMP_NOMINAL_C:		EQU		25

## Error Gain Factors

The control loop utilizes two gain factors to control the charging process. In general, the higher the error gain factors are set, the faster the error correction is initiated.  However, there is also a greater risk of instability.  It is suggested to set V_COR and I_COR as low as practical for acceptable charging performance to maintain a good stability margin.

### Current Error Gain Factor (I_COR)

I_COR stands for Current Correction.  This is the error gain factor used to amplify the present delta between measured current and it’s prescribed set point.  The higher the setting, the more the delta will be amplified. It is suggested to set I_COR as low as practical for acceptable charging performance to maintain a good stability margin.

### Voltage Error Gain Factor (V_COR)

V_COR stands for Voltage Correction.  This is the error gain factor used to amplify the present delta between measured voltage and it’s prescribed set point.  The higher the setting, the more the delta will be amplified. It is suggested to set V_COR as low as practical for acceptable charging performance to maintain a good stability margin.

## Special Installation Values and Flags

The values and flags listed below are used for unique or specific applications.  If any of the information seems unfamiliar, chances are they do not apply to your specific installation or application.

### Amp Hour Maximum Limit (AMPHR_MAXLIMIT)

This represents the maximum cumulative amphours allowed to be returned to the battery pack during a charging cycle.  If this limit is exceeded during the charging cycle, Fault Mode is entered.

Valid Range		=	1 to 255 Amp Hours

Normal Value	=	130 percent of Battery Pack Amp Hour Capacity

AMPHR_MAXLIMIT:	EQU		{{130*AMP_HOURS}+50}/100	;Max Amp	Hour Limit

### Dolphin Monitoring (MON_DOLPHIN)

The Dolphin unit, if installed, may be monitored via additional hardware for various fault conditions.  If the Lid is opened, the vehicle key turned on, the Dolphin contactor failed, or the FET turned off, the Dolphin pre-charge circuit may be damaged if charging continues.  If any of the mentioned failures are detected, charging will be terminated and Fault Mode will be entered. The use of the additional hardware and software monitoring prevents this damage from occurring.  Set this flag to a 1 (one) only if the additional hardware has been installed in both the vehicle and charge controller to monitor the Dolphin unit.

Valid Range    =  0 = Don't perform additional Dolphin monitoring.

=  1 = Perform additional Dolphin monitoring.

Default Value  =  1 = Perform additional Dolphin monitoring.

MON_DOLPHIN:    EQU     1                 ;Monitor Dolphin Unit Status

### Auxiliary Battery Monitoring (MON_AUX_BATT)

The Auxiliary Battery voltage may be monitored via additional hardware for state of charge.  The auxiliary battery may be damaged if over charged or the state of charge falls to low.  If the auxiliary battery voltage exceeds 14.3vdc or falls below 11.0vdc, charging will be terminated with a fault mode.  Set this flag to a 1 (one) only if the additional hardware has been installed in the charge controller to monitor the Auxiliary Battery.

Valid Range    =  0 = Don't perform Auxiliary Battery monitoring.

=  1 = Perform Auxiliary Battery monitoring.

Default Value  =  1 = Perform Auxiliary Battery monitoring.

MON_AUX_BATT:   EQU     1                 ;Monitor Dolphin Unit Status

### Perform Battery Leveling (DO_TOP_OFFS)

The DO_TOP_OFFS flag indicates the desire to perform traction pack and auxiliary battery topping off once Completion Mode has been entered.  If the flag is set, traction pack topping off voltage will be monitored every hour and auxiliary battery topping off voltage will be continuously monitored. Should pack voltage be at or below the level specified in V_START_LEVEL (see MODE 4 parameters), Mode 2 will be reentered and leveling will occur.  If the auxiliary battery voltage falls to 12.4 volts or less, Mode 4 will be entered.  If the flag is set to a zero, no topping off will occur.

Valid Range		=	0 = Don't perform traction pack and auxiliary battery topping off.

=	1 = Do perform traction pack and auxiliary battery topping off.

Default Value	=	1 = Do perform traction pack and auxiliary battery topping off.

DO_TOP_OFFS:   			EQU		1		;Do perform traction pack and auxiliary

;battery topping off.

### Level Traction Pack After n Cycles (LEVEL_AFTER)

The LEVEL_AFTER value indicates the required number of successful charge complete cycles which must be performed before a mandatory traction pack leveling cycle will occur.  Consecutive charge complete cycles is not required. Once this value has been reached, the charge controller will enter Mode 3 and begin leveling the traction pack..

Valid Range		=	0 to 254 cycles

Default Value	=	5 cycles

LEVEL_AFTER:    				EQU		5		;Level Pack after 5 charge completes.

(1)  The present 6.6kw SCM is limited to 405V.

(2)  The present 6.6kw SCM is limited to 20A.

# Making Firmware Changes

While entering data and changing parameters, all numbers must be entered as integers, no fractions are allowed.  The following steps describe the process of making firmware changes to the source listing.

## Hardware Requirements

- An IBM or compatible personal computer with the U.S. version of Windows 3.1 and DOS.

## Software Requirements

- A system diskette with the appropriate source file.

- An ASCII text editor.

- Cross-16 Meta-Assembler Version 2.0 or Cross-32 Meta Assembler.

## Procedure

1.  Load the system diskette and Assembler to the desired drive to be used.  Ensure the name of the directory where the assembler was installed is also added to the AUTOEXEC.BAT file.

Note:	Any changes made to the AUTOEXEC.BAT file require the personal computer to be rebooted before the changes will take affect.  Do this now, if required.

2.  Make the desired change(s) to the source file <filename.asm> utilizing the information and data worksheet found in this programming guide.  It’s always a good idea to make a backup copy of the source file prior to making any changes.  It is strongly recommended to make this backup copy now, if you haven’t done so already.

When making changes to any program constants (i.e. min’s, max.’s, gains, set points, delays, etc.) you should look for the following format in the Customer.h file:

CONSTANT_NAME		EQU		value

V_SET							EQU		05ah

If you find a need to change either of the constants used to perform scaling functions for voltage or current (i.e. voltage may be presently set at .4867 and current may be set at 10.50) only one entry is required. Within the Customer.h file are two constants called VOLTS_SCALE and AMPS_SCALE.  These two constants are also passed to the diagnostics program and used to re-scale the variables back to a normalized state.  Note that the constants are not in fractional format but are in an integer representation.  Making changes to these constants and reassembling the program will put the changes into effect for all affected constants (i.e. VTRIP1, I2, etc.).  Refer to section 12 for further information.

When entering any value in hexadecimal format, be sure to add a leading zero and a trailing h.  This helps the assembler identify the type of value you have entered for the constant.  You may enter decimal values without special characters or annotations.  Decimal numbers are the preferred method.  When entering decimal numbers, do not add a leading zero to the number.  Doing so will cause the assembler to assume the number is an octal entry.

Do not change the program name or revision date located within the EV13xxx.ASM program source code.  These are found near the end of the program.  They are used to determine the current version and build date of the software should any problems arise.

At the beginning of the EV13xxx.ASM source code file are two entries called titl and page which are followed by data.  These entries are for the output listing <filename.lst> file and do not affect anything else. The titl entry causes the data following it to be displayed as the header on each output page.  The page entry controls the number of lines printed on each output page.

3.  Assemble the program by typing

ASM   filename <cr>

where filename is the name of the file you just saved, and <cr> is symbolic for a carriage return.  You should not enter the file extension.  For instance, if you just saved a file called EV13xxx.ASM, all you would have to enter is the following:

ASM   EV13xxx

Additionally, a small batch file has been included with the installation software.  The batch file called A.BAT may also be used to assemble the program instead of the ASM <filename> instructions mentioned above.  Depending on the assembler version being used, the batch file may need to be edited as it defaults to using the C32 Assembler.

When the assembly completes, make sure there are no errors reported.  The program must assemble error free. If there are errors, you can use the <filename.lst> file to scan for the errors and then go back to the recently saved source file and make the appropriate corrections.  For instance, if you just assembled a file called EV13xxx.ASM, all you would have to do is either print or view the contents of EV13xxx.LST.

4.  When the program assembles error free, the micro-controller can be programmed with the new file, <filename.bin> data.

# Programming the Micro-controller

The following steps describe the process of programming a micro-controller with an IBM or compatible personal computer.

## Hardware Requirements

- An IBM or compatible personal computer with an available serial port.

- A 9-pin, d-type male to 9-pin, d-type female null modem cable (see Figure 1).  There are other combinations of cables and adapters which may be used.  The basic requirement is a null modem cable connecting the personal computer to the Electric Vehicle charge controller.

- A 25-pin, d-type female connector supplying +12 vdc input power at pin 10 and the 12 vdc return at pin 22.  If the controller is already integrated on a vehicle, this connection is already present however, in order to turn on the controller, the charge paddle must be inserted into the charge port.

- Note:  Charger must be turned off!

- A 12 vdc, 1 amp power source. If the controller is already integrated on a vehicle, this power source should already be present by inserting the charge paddle into the charge port.

## Software Requirements

- A system diskette with the <filename.bin> version of your assembled file.  This diskette should also include the RTSLO, RTSHI, ROMLOAD, and LOAD1 files as well.

## Procedure

1.  Place the system diskette in the appropriate drive.

2.  Connect the personal computer communications port (serial port) to the 9-pin controller connector by using a null modem cable.  See Figure 1.

Figure 1

3.  Connect the charge controller to the 12 vdc , 1 amp power source via the 25-pin input power cable.  If the controller is already integrated on a vehicle, this connection should already be present.

4.  Type the following information from the DOS prompt:

RTSLO  -C<comport> <cr>

where <comport> is your communications port (Com port or serial port) that is connected to the null modem cable.  An example for using Com port 2 would be as follows:

RTSLO  -CCOM2  <cr>

5.  Turn the 12 vdc, 1 amp power source ON.  If already integrated on a vehicle, this connection is made by inserting the charge paddle into the charge port.  Make sure the charger circuit breaker is OFF before inserting the charge paddle.

6.  To program the micro-controller enter the following information at the DOS prompt:

LOAD1  -F<filename.bin>  -C<comport>	<cr>		<cr>

An example for loading EV13xxx via Com port 1 would look like this:

LOAD1   -FEV13xxx.BIN   -CCOM1		<cr>		<cr>

Note that there are no spaces between the -F and the filename EV13xxx.BIN.

7.  Once you have entered the command line at the DOS prompt, you should see quite a few lines of data scrolling, followed by a “Download Complete” message.  If the data does not start scrolling, or if the data does not match an internal check, you’ll receive an abort message (within approximately 30 seconds) and you’ll need to start over again from step 1 of this procedure.

8.  At this point, the programming of the micro-controller is complete.  Turn the 12 vdc, 1 amp power supply OFF or remove the charge paddle.  Remove the 25-pin connector from the charge controller if performing this operation off-vehicle, and disconnect any remaining cables.

# Charge Control Algorithm Worksheet

PROGRAM NAME:												SOFTWARE REVISION:

TYPE OF BATTERY:											BATTERY CONFIGURATION:

CUSTOMER.H FILENAME:										TYPE OF VEHICLE:

|  |  |  |  |  | MODE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 254 | 255 |  |  |
|  |  |  | CV | CV | CV | CV |  |  |  |  |
| C U R R E N T | 20 -- 15 -- 10 -- 5 -- | P O W E R U P D E L A Y |  |  |  |  | C H A R G E C O M P L E T E | F A U L T E N C O U N T E R E D | - 400 - 375 - 350 - 325 - 300 - 275 | V O L T A G E |
|  |  | PUD | T1 | T2 | T3 | T4 |  |  |  |  |
|  |  |  |  | TMAX |  |  |  |  |  |  |

| POWER-UP DELAY | = | # OF BATTERY STRINGS | = | TEMP COMP. FLAG | = |
| --- | --- | --- | --- | --- | --- |
| TIME MAXIMUM (TMAX) | = | # OF CELLS PER BATTERY | = | TEMP MAXIMUM VOLTS | = |
| BATTERY AMP HOURS | = | BATTERY MAX VOLTAGE | = | TEMP MINIMUM VOLTS | = |
| # BATTERIES PER STRING | = | BATTERY MIN VOLTAGE | = | TEMP COMP MILLIVOLTS | = |
| AMP HOUR TRACKING | = | AMP HOUR TRIP AT (%) | = | TEMP NOMINAL DEG. C. | = |
| MON_AUX_BATT FLAG | = | MON_DOLPHIN FLAG | = | DO_LEVELING | = |
| V_START_LEVEL | = | LEVEL_AFTER | = |  | = |

| MODE | V1 (VOLTAGE SET POINT) | = |  |  | MODE | V2 (VOLTAGE SET POINT) | = |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | ITRIP1 (CURRENT TRIP) | = |  |  | 2 | ITRIP2 (CURRENT TRIP) | = |  |  |
|  | T1_HOURS | = | T1_MIN = |  |  | T2_HOURS | = | T2_MIN = |  |
|  | IMAX_MODE1 | = |  |  |  | IMAX_MODE2 | = |  |  |
|  | BYPASS_TRIPPT1 | = |  |  |  | BYPASS_TRIPPT2 | = |  |  |
| MODE | V3 (VOLTAGE SET POINT) | = |  |  | MODE | V4 (VOLTAGE SET POINT) | = |  |  |
| 3 | ITRIP3 (CURRENT TRIP) | = |  |  | 4 | ITRIP4 (CURRENT TRIP) | = |  |  |
|  | T3_HOURS | = | T3_MIN = |  |  | T4_HOURS | = | T4_MIN = |  |
|  | IMAX_MODE3 | = |  |  |  | IMAX_MODE4 | = |  |  |
|  | BYPASS_TRIPPT3 | = |  |  |  | BYPASS_TRIPPT4 | = |  |  |

DO NOT CHANGE THE FOLLOWING UNLESS REQUIRED

| V_COR | = 4 | I_COR | = 1 |
| --- | --- | --- | --- |
| VSCALE FACTOR | = .4867 | ISCALE FACTOR | = 10.50 |

# Inventory listing of the Installation Distribution Media

The files that are contained on the system installation disk are listed below.

| 68hc11.tb_ | 68hc11 op code instruction file. |
| --- | --- |
| a.ba_ | Batch file to optionally assemble the source code. |
| asm.ba_ | Assembler batch file. |
| ctl3dv2.dll | Installation Dynamic Link Library. |
| customer.h_ | Variables specific to this customer. |
| dosprmpt.pi_ | Program Information File (pif) to shell out to DOS from Windows. |
| equates.h_ | Defines 68hc11 register offsets |
| ev13172.ex_ | Electric Vehicle diagnostics interface program. |
| ev13172.as_ | Source code file for electric vehicle charge controller. |
| install.exe | Installation program. |
| load1.ex_ | Downloads romload.bin file to micro-controller ram, and desired binary file to EEPROM. |
| p.ba_ | Batch file to optionally begin programming the micro-controller. |
| prg13172.do_ | Electric Vehicle Charge Controller Firmware Programming guide, Microsoft Word 6.0 format |
| ramequs.h_ | Defines variables used in RAM. Do not change the sequence of these variables. |
| readme.wri | Changes for this release, Microsoft Write format |
| romload.bi_ | Micro-controller boot loader program. |
| rtshi.ex_ | Sets the micro-controller for single chip mode. |
| rtslo.ex_ | Sets the micro-controller for programming mode. |
| strip.ex_ | Pads empty EEPROM locations with FF hex, called from asm.bat |
| useguide.do_ | Electric Vehicle Diagnostic and Display Software User Guide, Microsoft Word 6.0 format |
| winstalc.dll | Installation Dynamic Link Library. |
| winstall.hlp | Installation help file. |
| winstall.inf | Installation information program. Do not change any data. |
| winstalp.dll | Installation Dynamic Link Library. |
| winstalz.dll | Installation Dynamic Link Library. |

Note:  Files ending with an underscore ( _ ) are compressed files.  The installation program will automatically decompress these files.

# Point of Contact

If there are any questions regarding the EV13xxx charging control program, please direct inquiries to:

Jeff Taylor

Hughes Power Control Systems

Building 237, Mail Station 1528

PO Box 2923

Torrance, CA  90509-2923

United States

Phone:	(United States)		(310) 517-5769

FAX: 	(United States)		(310) 517-5739 or (310) 517-5766

Email on the Internet:			TaylorJ@PCSSMTP.hac.com