ELECTRIC VEHICLE

DIAGNOSTIC AND DISPLAY SOFTWARE

USER GUIDE

HUGHES PROPRIETARY

Copyright © Hughes Aircraft Company (1994-1995)

- unpublished work - All Rights Reserved

HUGHES POWER CONTROL SYSTEMS

PO BOX 2923

TORRANCE, CA  90509-2923

November 2, 1995

1. Purpose

2. Requirements

2.1 Hardware Requirements

2.2 Software Requirements

3. Procedure

3.1 Installation and Update

4. Operation

4.1 Hardware Setup

4.2 Software Operation

4.2.1 File Command

4.2.1.1 New EV Log

4.2.1.2 Open Existing Log

4.2.1.3 Close EV Log

4.2.2 View Command

4.2.2.1 Locked / UnLock Counter

4.2.3 Action Command

4.2.3.1 Connect

4.2.3.2 Disconnect

4.2.3.3 Review Log

4.2.4 Settings Command

4.2.4.1 Communications Port.

4.2.4.2 Log Sample Rate

4.2.4.3 Default Button

4.2.5 Display Command

4.2.5.1 Active Log Stats

4.2.5.2 Battery Data

4.2.5.3 Badicheq Data

4.2.5.4 EPTI Data

4.2.5.5 Flags

4.2.5.6 J1850

4.2.5.7 Miscellaneous Information

4.2.5.8 Overall Charging

4.2.5.9 Port Status

4.2.5.10 Raw Data

4.2.6 Help Command

4.2.6.1 About

4.3 Other Helpful Information

5. Troubleshooting

5.1 Data Not Being Updated on the Screen

5.2 Data on the Screen is Erratic

5.3 Floating Point Error Message is Displayed

6. Inventory listing of the Installation Distribution Media

7. Reference

8. Point Of Contact

# Purpose

The purpose of this document is to provide the necessary information to install, update, and operate the Electric Vehicle (EV) charge controller diagnostic software.  This document does not provide instructions on performing changes to the charge controller firmware.  See the appropriate Programming Guide for information and instructions to perform firmware changes.

Throughout this document it is assumed the user has some background in Microsoft Windows and Disk Operating Systems (DOS).  Some of the examples in this document provide explicit details, but are not necessarily provided as the only way to accomplish the task.  They are provided for users who may be lacking experience in a particular area.  Experienced users will find it relatively simple to install or update the software in a variety of different ways.

This document is applicable to all versions of the Power Control Systems EV firmware released after 1 November 95.  There may be sections in this document which are not applicable to the accompanying version of software.  Specific examples are display screens for the Badicheq and EPTI charging systems.

# Requirements

The following sections define hardware and software requirements for the use and operation of the EV diagnostic and display software.

## Hardware Requirements

- An IBM or compatible personal computer with an 80386 processor (or higher), 640 kilobytes (K) of conventional memory plus 1024K of extended memory (4 megabytes or more is recommended), 1 megabyte of free disk space (10 megabyte recommended), a 3.5 inch high density floppy disk drive and an available 9-pin serial port.

- A 9-pin, d-type male to 9-pin, d-type female null modem cable (see Figure 1).  There are other combinations of cables and adapters which may be used.  The basic requirement is a null modem cable connecting the personal computer to the EV charge controller.

- A display adapter that is supported by Microsoft Windows 3.1.

- A mouse device that is supported by Microsoft Windows 3.1 is highly recommended. Although a mouse is not mandatory, it is recommended for ease in user interface with the EV program.

## Software Requirements

- An installation diskette with the EV diagnostic and display program.

- Disk Operating System (DOS) compatible with Microsoft Windows 3.1.

- Microsoft Windows 3.1.

Figure 1

Null Modem Cable

# Procedure

The following procedure explains how to install the EV diagnostic and display software for the first time.  The same procedures are used to install updates. While these procedures provide some explicit detail, variations may be used by the user, based on personal preferences and personal computer operating system literacy.

## Installation and Update

Start Windows following the normal procedure.  Insert the floppy disk labeled Installation Disk into the appropriate floppy drive.  From the Program Manager or File Manager menu, select File and then Run.  Enter the following command:

A:\INSTALL.EXE		if using drive A:

or

B:\INSTALL.EXE		if using drive B:

At this point the installation program will start.  The opening screen will provide an option to set the Source and Target directories, Create Program Manager Group, and whether or not to install the Program Files.  Normally the default options and selections should be acceptable.  If changes are desired, make the appropriate change.  The Help button provides general information about each available option.  When satisfied with all of the options, select Start Install to being the installation and / or update.

During the installation process, the EV Diags window group will be created, if not already available and the Create Program Manager Group option was selected.  Six (6) icon’s will be added automatically and are as follows:

| EV Diags Program | DOS Prompt | Changes to this Version |
| --- | --- | --- |
| EV Information File | Programmers Guide | User Guide |

If previous versions of the files being installed are found in the target directory, a prompt will appear to allow the option to overwrite or skip the duplicate file.

Upon completion of the installation process, a dialog box will appear and indicate the successful completion.  If during the installation process duplicate files were found, and the option to not overwrite the files was selected, an unsuccessful completion message may appear.

Note: If installing an updated version of the diagnostic program, it is recommended  to keep a copy of the previous release for use at a later time, should the need arise.  You can determine the version number by starting the diagnostic and display program under Windows and selecting Help / About.  At this point in time, the updated programs are not 100% backward compatible.

Once the installation is complete, be sure to review the Readme.WRI file for information concerning changes to the programs.

If this installation was an update, normally the updated files will be installed to a new directory.  This protects the integrity of your existing data.  The new Customer.h file will need to be edited to reflect the desired data and parameter settings.  Do not use any previous version of the Customer.h file as there may be incompatibilities.

# Operation

The following paragraphs detail the operation of the EV diagnostic and display program.  Each menu item is fully described and as well as the expected results.

## Hardware Setup

Connect the personal computer serial communications port to the 9-pin charge controller connector, via a null modem cable.  A schematic for the null modem cable is provided in Figure 1.

Start Windows if not already running, and select the appropriate diagnostics icon in the EV Diags group window.  Double click on the icon and the diagnostic program will begin.

The main screen for the diagnostic program will immediately display. Data will appear next to the parameters after the charge controller has completed the power-up delay (charge paddle must be inserted), and the data output from the charge controller is in synchronization with the personal computer.

Select the Settings command at the top of the EV Diags screen and ensure the proper Communications Port is selected for the personal computer.  Options to select from Com Port 1 through Com Port 4 are available.  After selecting the desired communications port, select OK to close the window.

If after selecting OK for the communications port window a message is displayed indicating the port could not be opened, try selecting another port.  The message indicates either the selected port is already busy, does not exist, or has a deficiency.

Note:  This release of the Windows diagnostic program, will always revert to communications port 1 when starting the program.  Future releases of the diagnostic program will have a save feature for the communication settings.

Select the Action menu command, Connect option, to start the diagnostics program.  The program will immediately seek a synchronization with the charge controller and then begin displaying data.  Remember, the paddle must be inserted into the charge port for the controller to operate.  If the paddle is not inserted, data will not begin displaying on the screen.

## Software Operation

The following sections breakout the commands available at the top of the display screen.

### File Command

The File Command provides a data interface between the data packets received from the charge controller and the disk drive.  Options are provided to allow new data logging, extracting previously saved data, closing an open data log, or exiting the EV diagnostic and display program.

#### New EV Log

This command will prompt for a filename to save the data to the disk as it is received from the charge controller.  This operation works asynchronously with the program.  As data is received from the charge controller, and the data is in synchronization with the personal computer, the data is written to disk.  Each packet of information received is saved until either the Close EV Log or Exit command is selected.

Note:	Caution should be used when logging data.  As data is received from the charge controller, disk space is used to store this data.  A prolonged period of data logging may quickly utilize remaining free space on the personal computer disk drive.  See the Settings command.

It is recommended to use filenames with a suffix of EV1.  The EV portion of the suffix implies Electric Vehicle.  The number 1 is to imply this is the first generation of data logging for the EV charge controllers.  Once the filename is either entered or selected, the file will be opened and the program will return to the main screen.  A file may be opened while already connected to the charge controller, or before a data connection is completed.

#### Open Existing Log

This command allows the retrieval of previously logged data only. This command is not used to begin logging data, but for reviewing existing data.  A prompt for the filename will appear with a default file filter of *.EV*.  The file filter allows selection of existing EV data log files.  Once the file is selected, the file will be opened and the program will return to the main screen.  If a data connection already exists (data being received from the charge controller), this command will not be available.  It will appear as a grayed menu item.  A data connection is not allowed while an existing file has been opened for review.  This prevents existing data from being overwritten by new data.

#### Close EV Log

This command closes an open data logging file, whether it is a data log opened by the New EV Log or Open Existing Log command.  This command is inactive (grayed) when a file is not presently open.  When the file is closed, it will use the same name that was used for the file with either the Open File command or the New File command.

### View Command

When this menu item is selected, a sub-menu is displayed with an option to view the Locked / UnLocked Counter.

#### Locked / UnLock Counter

As a data link is established between the charge controller and the personal computer, a synchronization process must occur.  As the synchronization takes place, the synchronization is initially in an UnLocked state.  A counter tracks how many times the data packets are in (locked) and out (UnLocked) of sync.  This option, if checked, displays the counters which track the synchronization process.  It is not necessary for the counters to be displayed for the software to operate.  It’s merely an indication of the data link stability.

### Action Command

When this menu item is selected, a sub-menu is displayed with options to Connect, Disconnect, or Review Log.

#### Connect

The Connect command establishes a data link and synchronization between the charge controller and the personal computer.  When this command is selected, the screen will display an UnLocked message toward the bottom right side of the screen, provided the View command, Locked / UnLocked option is enabled.  As a data packet is received from the charge controller, a counter is optionally displayed next to the UnLocked message.  Once data synchronization is attained, a Locked message will be displayed and the data will begin appearing on the screen.  This command will be disabled (grayed) after being selected and will remain disabled until the Action / Disconnect command is selected.  See the View command for additional information concerning the Locked / UnLocked counters.

Note:  If a file logging was in progress, and Disconnect was selected, and the file was not closed prior to selecting Disconnect, upon reselecting Connect, the file logging process will resume.

#### Disconnect

This command terminates the data link between the charge controller and the personal computer.  When the command is selected, data on the screen will freeze and the Locked / UnLocked counters will no longer update.  This command is disabled (grayed) at program start-up until the Connect command is selected.  See the View command for additional information concerning the Locked / UnLocked counters.

Note:  If a file logging was in progress, and Disconnect was selected, and the file was not closed prior to selecting Disconnect, upon reselecting Connect, the file logging process will resume.

#### Review Log

This command is used to review data which has been previously saved to a data log file.  Use of this command is in conjunction with the File / Open Existing Log command.  Once selected, the data log is scanned to determine the available data.  Upon completion of the data scan, a button bar is displayed with options for reviewing the data.  This command is disabled (grayed) unless a data log file has been opened by using the File / Open Existing Log command and the file was opened successfully.  Access to this command is prohibited while a data link is established.

### Settings Command

This command allows the user to select the desired Communications Port and Log Sample Rate, and to restore the Default Settings.

#### Communications Port.

The default setting for the communications port is number 1.  Selecting the scroll button on the right side of the display box will allow selection of any communications port from 1 through 4.  The selected port must be available for use in receiving data via the null modem cable.

#### Log Sample Rate

This option determines the percentage of data packets received from the charge controller data port, to be saved to disk.  The default setting is for 100% of the received data or, in other words, every packet of received data would be saved to disk.  This setting has no affect on the operation of the program unless the File / Open option was successfully executed and the file is still open.  Then data will be logged to the disk.  Selecting the 50% option would yield every other data packet being saved to the disk.

#### Default Button

Selecting the Default button will cause all previously selected settings to revert to their default state.  The default states are described in the above sections.

### Display Command

When this menu item is selected, a sub-menu is displayed with options to display Active Log Stats, Battery Data, Flags, J1850, Miscellaneous, Overall Charging, Port Status, and Raw Data information.

#### Active Log Stats

A logged data file must be open for this command to be enabled. This command option will display the statistics of an opened logged data file. Information is presented concerning each mode of logged data available for review.  This includes the number of records per mode and total records in the data file.  This command is disabled while the personal computer is receiving data from the charge controller and the Action / Connect command is selected.

#### Battery Data

This command displays information specific to the battery.  Available information includes what was coded by the user and the present status of the battery.  The display is continuously updated during the charging process when the Connect command on the Action menu was successfully implemented. When the controller is operational (i.e. charge paddle inserted and system charging) information may be monitored which could provide insight as to why the system may not be functioning properly.

#### Badicheq Data

This command displays information specific to a Badicheq pulse charging installation and operation.  Data transmitted and received from the Badicheq unit is presented for review by the user.  Available information includes pulse widths, frequency, calculated charge commands and more.  The display is continuously updated during the charging process when the Connect command on the Action menu was successfully implemented. When the controller is operational (i.e. charge paddle inserted and system charging) information may be monitored which could provide insight as to why the system may not be functioning properly.

#### EPTI Data

This command displays information specific to an EPTI charging interface. Data transmitted and received from the EPTI controller includes the charge command, duration, checksum and more.  The display is continuously updated during the charging process when the Connect command on the Action menu was successfully implemented. When the controller is operational (i.e. charge paddle inserted and system charging) information may be monitored which could provide insight as to why the system may not be functioning properly.

#### Flags

This command displays information about the status of various flags within the charge controller program. The display is continuously updated during the charging process when the Connect command on the Action menu was successfully implemented. When the controller is operational (i.e. charge paddle inserted and system charging) flags may be monitored which could provide insight as to why the system may not be functioning properly.

#### J1850

This command displays information about the status of the J1850 communications between the charge controller and the charger. This information is continuously updated during the charging process when the Connect command on the Action menu was successfully implemented.  Monitoring of each communication device is provided as well as overall system communication.  Some fields on this screen are updated about once every ten seconds. When the controller is operational (i.e. charge paddle inserted and system charging) information may be monitored which could provide insight as to why the system may not be functioning properly.

#### Miscellaneous Information

This command displays miscellaneous information about the status of the charge controller.  Presently, data includes information about the brown-out monitoring flags and variables.  Currently it is used for tracking variables during the development of changes to the diagnostic program.  Most of the information on this screen is of little value to the customer at this time. The display is continuously updated during the charging process when the Connect command on the Action menu was successfully implemented. When the controller is operational (i.e. charge paddle inserted and system charging) flags may be monitored which could provide insight as to why the system may not be functioning properly.

#### Overall Charging

This command displays information concerning the present charging cycle. This is perhaps the most used screen since it contains a wealth of performance data. The display is continuously updated during the charging process when the Connect command on the Action menu was successfully implemented. When the controller is operational (i.e. charge paddle inserted and system charging) flags may be monitored which could provide insight as to why the system may not be functioning properly.

#### Port Status

This command displays information concerning the present status of the four ports on the micro-controller.  Not all data lines on each port are used.  Some are used for discrete inputs and outputs. Data signals may be inverted due to software requirements so exercise caution when trying to diagnose malfunctions. The display is continuously updated during the charging process when the Connect command on the Action menu was successfully implemented. When the controller is operational (i.e. charge paddle inserted and system charging) flags may be monitored which could provide insight as to why the system may not be functioning properly.

#### Raw Data

This command displays each packet of data as it is received from the charge controller.  Data is displayed in it’s unmodified raw format. Use of the Ramequs.h file and this screen may provide insights as to micro-controller performance and calculations. The display is continuously updated during the charging process when the Connect command on the Action menu was successfully implemented. When the controller is operational (i.e. charge paddle inserted and system charging) flags may be monitored which could provide insight as to why the system may not be functioning properly.

### Help Command

When this menu item is selected, a sub-menu is displayed with an option to display the About information.

#### About

This command will display the present version number and build date of the EV diagnostic and display program.

## Other Helpful Information

The diagnostic program may be terminated by using Alt-F4, or selecting the Action command, followed by the Exit option.  It is not necessary to select Disconnect prior to selecting Exit.  If a data file is open, it will be closed prior to exiting the diagnostics program. The Disconnect option at may be used at any time to stop the screen from updating.  Should the Connect option then be selected, the program will once again go through a synchronization process.

Note:  If a file logging was in progress, and Disconnect was selected, and the file was not closed prior to selecting Disconnect, upon reselecting Connect, the file logging process will resume.

# Troubleshooting

The following section is intended to assist in determining why difficulties may be experienced in the utilization of the diagnostic and display program.

## Data Not Being Updated on the Screen

Verify the Action / Connect command is grayed.  If it’s grayed, it means the command is active.  If not grayed, select the Action / Connect command to begin the data connection.  When the Action / Connect command is grayed, the Settings command at the top of the screen should also be grayed.  If the Action / Connect command is was already grayed, try selecting Disconnect and then Connect.

Verify the null modem cable is connected and wired properly.  See Figure 1 for a pin-out of the cable.

Charge paddle is not inserted into the charge port or the charge port is not seeing a connection present signal.

SCM is not turned on.

SCM has a fault message displayed.

## Data on the Screen is Erratic

This may be occur when the mouse is moved.  If so, try utilizing another video driver for Windows.  The standard Windows VGA driver seems to work best.  The video driver may be changed by using the Windows Setup program.

There may be a conflict with the communications port interrupt.  Interrupts are critical to proper operation of any computer.  Utilizing the Windows Control Panel, the interrupts may be examined and changed through the Ports icon.  Microsoft DOS also has the MSD program available to examine the computer configuration.

Check for a loose or defective null modem cable and / or connection pins.

## Floating Point Error Message is Displayed

If this occurs, please note the Display window which is in use and notify the point of contact listed at the end of this document.  The problem is internal to the diagnostic program.  Every effort has been made to eliminate this type of error.

# Inventory listing of the Installation Distribution Media

The files that are contained on the system installation disk are listed below.  Filenames containing an x indicate the filename is dependent upon the accompanying software.  Examples are EV1xxxx.as_ and EV1xxxx.ex_.

| 68hc11.tb_ | 68hc11 op code instruction file. |
| --- | --- |
| a.ba_ | Batch file to optionally assemble the source code. |
| asm.ba_ | Assembler batch file. |
| ctl3dv2.dll | Installation Dynamic Link Library. |
| customer.h_ | Variables specific to this customer. |
| dosprmpt.pi_ | Program Information File (pif) to shell out to DOS from Windows. |
| equates.h_ | Defines 68hc11 register offsets |
| ev1xxxx.ex_ | Electric Vehicle diagnostics interface program. |
| ev1xxxx.as_ | Source code file for electric vehicle charge controller. |
| install.exe | Installation program. |
| load1.ex_ | Downloads romload.bin file to micro-controller ram, and desired binary file to EEPROM. |
| p.ba_ | Batch file to optionally begin programming the micro-controller. |
| prg1xxxx.do_ | Electric Vehicle Charge Controller Firmware Programming guide, Microsoft Word 6.0 format |
| ramequs.h_ | Defines variables used in RAM. Do not change the sequence of these variables. |
| readme.wri | Changes for this release, Microsoft Write format |
| romload.bi_ | Micro-controller boot loader program. |
| rtshi.ex_ | Sets the micro-controller for single chip mode. |
| rtslo.ex_ | Sets the micro-controller for programming mode. |
| strip.ex_ | Pads empty EEPROM locations with FF hex, called from asm.bat |
| use1xxxx.do_ | Electric Vehicle Diagnostic and Display Software User Guide, Microsoft Word 6.0 format |
| winstalc.dll | Installation Dynamic Link Library. |
| winstall.hlp | Installation help file. |
| winstall.inf | Installation information program. Do not change any data. |
| winstalp.dll | Installation Dynamic Link Library. |
| winstalz.dll | Installation Dynamic Link Library. |

Note:  Files ending with an underscore ( _ ) are compressed files.  The installation program will automatically decompress these files.

# Reference

- DOS operating guide

- Microsoft Windows 3.1 operating guide

# Point Of Contact

If there are any questions or suggestion regarding these procedures, please direct inquires and comments to:

Jeff Taylor

Hughes Power Control Systems

Building 237, Mail Station 1528

PO Box 2923

Torrance, CA  90509-2923

United States

Phone:  (United States)	(310) 517-5769

FAX:  	   (United States)	(310) 517-5739 or (310) 517-5766

Email on the Internet:	TaylorJ@PCSSMTP.hac.com