Protect that Dolphin Need 6 fuse holders, 6 ea AGC1 fuses. Install 4 in the Dolphin and 2 in the gray junction box. Take a look at the charger board. It has a 13 pin connector on it with a harness that leads to the connector at the back-center of the main board. Notice the pattern of the wires in the charger connector. (1)--(4)--(7)(8)-(10)(11)(12)(13). The dashes are empty. Start counting from the position that is closest to the front of the truck. Add 1 amp, 250vac fuses to positions 1, 4. Then add 1 amp fuses to positions 7, 8. One and four is the AC voltage going to the main board. Seven and eight is the pwm that goes from the main board to the charger. Or The last two, 1 amp fuses go in the junction box. One comes from the KEY switch and CP, because the charge port also can send wild +12V into the Dolphin. Or