BUSCOM User’s Manual

Author: Brad Rutledge

Rev Date: 2/22/98

Overview

BUSCOM is a software upgrade of the DANCOMM diagnostic software used with the Dolphin 65 and Dolphin 120 Power Conversion Units (PCUs). Whereas the previous DANCOMM software was composed of multiple programs, one for each diagnostic display, and was “tuned” to a unique PCU software version, BUSCOM is a single program that can switch between multiple diagnostic displays and can be used with any PCU software version, i.e. it is forwards and backwards compatible. Like DANCOMM, BUSCOM provides the user with two standard displays, a virtual dashboard display and a propulsion fault display. BUSCOM also provides the user with the option of using custom input display files to monitor specific PCU parameters that may or may not be included in the standard displays. BUSCOM also includes a self-test feature to check the PC-to-PCU communications link.

Hardware Setup

The hardware setup for BUSCOM is shown in Figure 1. BUSCOM executes on a PC (typically a laptop PC for portability reasons). The PC COM1 serial port provides an RS232 interface which must be connected to the mating 9-pin RS232 interface connector on an external RS232-to-J1850  “conversion box”, (sometimes called a “Dancomm box”), available through Raytheon Technical Services Company (RTSC). The two J1850 leads or connectors on the “conversion box” must be connected to the PCU’s J1850 interface, (PCU connector J4, pin ‘s’ - J1850 bus and pin ‘k’ - ground). The external “conversion box” has an On/Off power switch, (usually there is a red dot on the “On” side of the switch), and receives power via a standard 120 VAC plug. When external 120 VAC power is not available, the “conversion box” will also operate off its internal rechargeable 9 V batteries. Internal to the PCU, the J1850 bus is connected to the HC11 microprocessor. The HC11 microprocessor communicates with the TMS320 microprocessor via a separate Universal Asynchronous Receiver/Transmitter (UART) serial communications bus.

Notes:

1. To conserve internal battery power, the “conversion box” should be switched to the “Off” position when not in use. The internal batteries will be “trickle charged” whenever the “conversion box is connected to a 120 VAC power source. If the batteries are completely discharged, they may be recharged by plugging the “conversion box” into a 120 VAC source overnight (charging will be slightly more efficient with the On/Off switch set to the Off position). Do not leave the “conversion box” plugged into a 120 VAC source for extended periods of time, however, as this will overheat the internal batteries.

RS232

-to-

J1850

Conversion

Box

PC

RS232

COM1

Display

PCU

HC11

J1850

TMS320

UART

Ground

120 VAC

Power

Figure 1. BUSCOM Hardware Setup

How To Run BUSCOM

BUSCOM executes from a DOS environment. To run BUSCOM, change to the directory containing the executable BUSCOM program (BUSCOM.EXE) and the input display files, (e.g. DASH_D.SCN, FAULT_D.SCN, etc.), and type the following command at the DOS prompt:

buscom <v>

where <v> = optional PCU software version specifier (e.g. ‘c’, ‘d’, etc.)

For example, the command ‘buscom c’ would invoke buscom and direct the program to use the standard Rev. C input display files (DASH_C.SCN and FAULT_C.SCN).

Notes:

1.  If the optional PCU software version is not specified in the command line, the program defaults to Rev. D PCU software compatibility.

2.  DOS does not distinguish between lower and upper case characters with respect to the command ‘buscom’ or the optional PCU software version specifier.

Input Display Files

The input display files for the appropriate PCU software version must be located in the same directory as BUSCOM.EXE. The standard input display files must follow the following naming convention:

dash_x.scn = virtual dashboard display for PCU software version ‘x’

fault_x.scn = propulsion fault display for PCU software version ‘x’

where ‘x’ = PCU software version (e.g. ‘c’, ‘d’, etc.)

BUSCOM also allows custom input display files created by the user to monitor specific parameters which may or may not be included in the standard input display files. Custom input display file names do not have to follow the above naming convention but are limited to 32 characters in length including the extension. It is suggested that the extension ‘.SCN’ (an abbreviation for “screen”) be used for custom input display files but this is not a requirement.

Input Display File Format

The standard input display files contain data item description information corresponding to parameters to be monitored in the HC11 or TMS320 microprocessors in the PCU. Each data item in the file has a short name followed by data item description information encoded in the following format:

[loc, addr, format, bitpos

where

‘[‘ = marker indicating start of data item description information

loc = location of data (‘T’ = TMS320, ‘H’ = HC11, ‘X’ = fault words)

addr = 4-digit absolute hex data address with leading zeroes (when loc = ‘T’ or ‘H’),

1-digit decimal fault word number (when loc = ‘X’)

format = display format descriptor (‘A’, ‘B’, ‘C’, ‘F’, ‘K’, ‘P’, ‘R’ or ‘U’)

bitpos = decimal bit position for single-bit data (0 - 15)

Data locations must be upper case characters. Data locations ‘T’ and ‘H’ are used in the standard virtual dashboard input display files and may be used in custom input display files. Data location ‘X’ is used exclusively in the standard propulsion fault input display file and is not allowed in custom input display files. BUSCOM automatically retrieves propulsion fault words from the HC11 where they are stored in compressed format at reserved locations in HC11 memory.

When the data location is ‘T’ or ‘H’, BUSCOM interprets the data address as a 4-digit absolute hex address of the data in the TMS320 or HC11 microprocessor RAM. Valid hex addresses for the TMS320 RAM are 0000 - 00ff. Valid hex addresses for the HC11 RAM are 2000 - 22ff.

When the data location is ‘X’, BUSCOM interprets the data address as a 1-digit decimal fault word index. While there are 7 fault words in the TMS320, valid fault word indices are only 0 - 3. This is because the 7 TMS320 fault words are compressed into 4 fault words in the HC11.

Display format descriptors must be upper case characters. The following is a list of valid display format descriptors and their definitions:

‘A’ = 8-bit aux. batt. A/D data displayed as a scaled float (0 - 0.0 V, 255 = 15.25 V)

‘B’ = single-bit positive logic (0 = OFF, 1 = ON)

‘C’ = 8-bit raw data displayed as a 3-digit unsigned decimal with leading zeroes

‘F’ = fault bit data (displays ‘X’ if fault bit is set)

‘K’ = 10-bit acc. ped. A/D data displayed as a scaled float (0 = 0.0 V, 1023 = 5.0 V)

‘P’ = PRNDL position (text display)

‘R’ = 16-bit raw data displayed as a 6-digit decimal with leading zeroes

‘U’ = single-bit reverse logic (0 = ON, 1 = OFF)

Display format ‘C’ assumes that the 8-bit data is in the upper byte of the 16-bit integer data passed to the routine. This is consistent with char type variables sent from the HC11.

Display format descriptor ‘F’ is used exclusively when the data location is ‘X’, which occurs only in the standard propulsion fault input display file, and therefore may not be used in the standard virtual dashboard input display file or custom input display files.

Bit position is only applicable for single-bit data (display format descriptors ‘B’, ‘F’ and ‘U’). Bit position is ignored for all other types of data.

Notes:

1.  Commas are required as separators between data item description elements. A comma is not allowed between the start marker (‘[‘) and the first element.

- Spaces between data item description elements are not allowed.

Input display files also contain a reserved display line for error messages. This line reads:

ERROR MSGS: ####

BUSCOM searches for the string “####” in the input display file and will automatically display any error messages beginning at that location on the screen. BUSCOM also uses the 3 lines following the error message line to display diagnostic information and obtain user input where appropriate.

Creating Custom Input Display Files

Custom input display files must follow the formatting rules described in the section entitled “Input Display Files”. The easiest way to create a custom input file is to copy the virtual dashboard input display file and modify data item names and addresses as required. If it is desired to monitor propulsion faults as part of a custom display, the actual address of the corresponding fault word in either the HC11 or the TMS320 must be specified in the custom input display file along with the appropriate data item location identifier (‘H’ or ‘T’).

Up to 3 data items may be displayed on one line. The first data item name should start in column 1 and the corresponding data item description information should start in column 14. The second data item name should start in column 27 and the corresponding data item description information should start in column 40. The third data item name should start in column 53 and the corresponding data item description information should start in column 66. Violation of these screen formatting rules may result in parts of the screen being overwritten by BUSCOM under certain conditions.

BUSCOM always uses line 24 (the last line on the display screen) to display help information. Since BUSCOM requires an error message line and reserves 3 lines at the bottom of the screen for diagnostic information and user inputs, the maximum number of lines available to the user for display of data items is 19. Since a maximum of 3 data items may typically be displayed per line, this limits the total number of data items that may be displayed at one time to 57. In addition, there is normally a title line at the top of the screen which further reduces the total number of data items that may be displayed. If it is desired to display more than 57 data items, it is suggested that additional custom input display files be created.

Operation

When BUSCOM is invoked, the following message will appear on the screen:

PCU software version: Rev. v

Input file = dash_v.scn

Press [ENTER] to continue.

where v = specified PCU software version or ‘d’ by default

After the user presses [ENTER], the virtual dashboard display for the appropriate PCU software version will appear on the screen and BUSCOM will automatically begin updating the data item values on the screen. BUSCOM allows the user to easily move between the standard displays (i.e. the virtual dashboard and propulsion fault displays) and custom displays without exiting the program by typing a single-character command at the keyboard during program execution. The following are valid BUSCOM commands:

‘d’ = switch to virtual dashboard display

‘f’ = switch to propulsion fault display

‘c’ = switch to custom display where custom input display file will be supplied by user

‘t’ = perform PC-to-HC11 communications link self-test

‘ ‘ = clear all data item values on display

‘x’ = exit BUSCOM program and return to DOS

As a reminder to the user, BUSCOM automatically displays the following line of help information at the bottom of the screen showing the available BUSCOM commands:

<d=Dash> <f=Faults> <c=Custom> < =Clear> <t=Test> <x=Exit>

Notes:

1.  BUSCOM will respond to either lower or upper case character commands.

2.  Characters which do not correspond to valid commands are ignored.

3.  An additional function which “pokes” (writes) data to the TMS320 RAM in real-time exists but has been disabled in this release of BUSCOM due to the potential dangers involved in users inadvertently corrupting the TMS320 microprocessor’s memory.

Communications Self-Test

BUSCOM also includes a communications self-test feature which may be invoked at any time by entering the character ‘t’ at the keyboard while BUSCOM is running. This feature performs an “echo test” of the communications link between the PC and the HC11 and TMS320 microprocessors inside the PCU and reports results back to the user. It is recommended that the communications self-test be performed whenever communications between the PC and the PCU are suspect, for example, if error messages are displayed on the screen or data item values are questionable or seem to change excessively. Following the self-test, BUSCOM returns to the previous display screen and continues execution.

Troubleshooting Tips

There are two types of error messages: fatal error messages and non-fatal error messages. Fatal error messages are typically self-explanatory and generally cause the program to automatically exit. An example of a fatal error condition is when BUSCOM cannot find the appropriate input display file in the same directory as BUSCOM.EXE.

Non-fatal error messages are displayed on the error message line (typically near the bottom of the display) and are generally indicative of communication or timing errors. BUSCOM will continue to execute and display error messages as error conditions are encountered, (this may result in error messages flashing briefly on the screen). When this occurs, the user should invoke the BUSCOM communications self-test feature. If the communications self-test fails, check all connections between the PC and the PCU and the “conversion box” is turned on and has power. If these actions fail to rectify the problem, contact RTSC for further help. If the communications self-test passes and error messages continue to appear on the display, contact RTSC for further help.